Invacom F700245 30m FC/PC Pre-Terminated Fibre

3mm steel armoured single mode fibre optic cable UV stabilised, Gray PVC Outer, G657A - LSZH operating temp -0ºC….+60ºC

Kost al snel rond de €27.50 en €30-

Invacom F101588 Quad Optical Fibre GTU

GLOBAL INVACOM QUAD OPTICAL FIBRE GTU

The FibreMDU system distributes digital satellite signals to 32 points over a single fibre Passive Optical Network (PON) connected directly to an Optical LNB (FibreMDU LNB). An MDU Optical Convertor (Virtual Twin, Virtual Quad or Virtual Quatro) is located at each of the points connected, this converts the signals back to a form that all digital satellite STBs expect to receive. Each end user is able to receive all the services and programming offered by the broadcaster just as if they were connected to their own antenna.

Each of the Optical Convertors have been developed to replicate specific types of traditional LNB (Twin, Quad and Quatro) by converting the optical signal from the FibreMDU Optical LNB back to the satellite IF signal received at the antenna. The signals are output from the Optical Convertor via standard F connections, and are ready to be connected directly to STB tuners (Virtual Twin/Quad) or traditional multiswitches (Virtual Quatro).

The GI FibreMDU Virtual quad connects to a single fibre optic cable from a FibreMDU Optical LNB. This is done either through a network of passive optical splitters (PON) or directly from the LNB (with attenuation). Each Virtual Twin provides 4 universal feeds.

Features

- Converts optical signals from a FIBRE LNB to IF.
- Provides up to 4 universal satellite feeds from 1 fibre optic connection.
- Plug and Play.
- Powered via the STB.

Specifications

RF Frequency Range: 950~5450MHz
Optical RLR: 20dB
Aggregate Equivalent RF Power: -60dB~-20dB
SAT Transponders: 120
Nominal Impedance: 70Ohm

Kost ook al snel €120 a €130-

Invacom F925004 FibreMDU Optical Output LNB & PSU

GLOBAL INVACOM FIBRE MDU OPTICAL OUTPUT LNB

Features

- Converts all 4 universal IF bands to a single optical output (H/H - H/L - V/H - V/L = single optical output).
- Capable of supplying all converted signals to 32 distribution points spread over a 10 kilometre radius.
- 40 mm Feed Horn.

The GI FibreMDU Optical Output LNB radically changes how satellite IF signals are dealt with compared with conventional universal LNBs. This innovative new design uses patented technology to frequency stack both horizontal and vertical polarities, creating a single IF frequency range of 950MHz - 5,45GHz. The newly created single band is then frequency modulated optically and output using a 1310nm laser internal to the Optical Output LNB.

Powered separately the Optical LNB takes the required <450mA from a standard 12V supply utilising a standard female F type connector. Fibre connection is made via a standard FC/PC connector feeding the distribution network on single mode fibre optic cable. With an optical output +7dBm the GI FibreMDU Optical Output LNB is easily capable of driving up to 32 GI FibreMDU converters located within a 10km radius.

Specifications

Input frequency - 10,7 - 12,75 GHz (H and V)
Output frequency - 950MHz - 5,45GHz, modulated laser output 1310 nm
Optical output power - 7dBm nominal at 25°C, +/-2dBm over full temperature range
Noise figure - 0,5 - 1,3 dB (depends on ambient temperature)
Gain - 62 - 72 dB (at room temperature)
Gain variation - +/- 2dB (-30 - +60°C)
Gain flatness - 5dB per band (0,95 - 5,45GHz)
Gain ripple - The gain ripple per 26MHz bandwidth must be less than +/-0,5dB
Phase noise - Offset frequency - Maximum limit: 1KHz - -55dBc/Hz, 10KHz - -80dBc/Hz, 100KHz - -100dBc/Hz, 1MHz - -110dBc/Hz
Local oscillator stability - Condition - Max variation from nominal freq. Initial setting +/-1MHz. Temp. drift (-40 - +60°C) +/-2MHz. Aging and total drift (10 year life) +/-4MHz
Current consumption - < 450mA
Image rejection - 40dB min.
Cross polar isolation - min. 25dB, typ. 30dB
Output connectors - DC Input - F-connector (female), Optical output - FC/PC
Supply voltage - 12V
Spurious output - (after recovering modulation) in band (950MHz - 3GHz, 3,4GHz - 5,45GHz) -25dBc

Ook weer €120 a €130-

Dus het is er wel maar kost ook wat weet niet of dit op wegt tegen de voordelen.