Setup “static” smartreaders using udev

If we enter one (new) reader into our pc, in Linux it will show up as:

T:  Bus=04 Lev=01 Prnt=01 Port=01 Cnt=01 Dev#= 15 Spd=12  MxCh= 0 
 D:  Ver= 2.00 Cls=00(>ifc ) Sub=00 Prot=00 MxPS= 8 #Cfgs=  1 
 P:  Vendor=0403 ProdID=6001 Rev= 4.00 
 S:  Manufacturer=Argolis BV 
 S:  Product=Smartreader plus 
 S:  SerialNumber=Reader 0 
 C:* #Ifs= 1 Cfg#= 1 Atr=a0 MxPwr=100mA 
 I:  If#= 0 Alt= 0 #EPs= 2 Cls=ff(vend.) Sub=ff Prot=ff Driver=ftdi_sio 
 E:  Ad=81(I) Atr=02(Bulk) MxPS=  64 Ivl=0ms 
 E:  Ad=02(O) Atr=02(Bulk) MxPS=  64 Ivl=0ms

This reader we want to use only for our German card (caid = 0x1702) on a slightly higher clock rate

So first we rename the smartreader using the (new) srp_tools

./srp_tools.x86 –n G

So now the reader (after removing/reinserting) will show up as “Reader G”

Now it is time to make udev catch this reader and give it a more appropriate name, to do this we modify or add one new file into the /etc/udev/ (this can vary from distro !!)

SUBSYSTEMS=="usb", KERNEL=="ttyUSB*", ATTRS{serial}=="Reader G",  SYMLINK+="SmartReader+_1702"

So what do we get now when we insert the reader into our system ?

A syslink called  /dev/Smartreader+_1702

Now we remove the reader and put in reader 2

Frist we rename the reader again

./srp_tools.x86 –n N

So this one will be called “Reader N”

Again we add this reader into our udev config

SUBSYSTEMS=="usb", KERNEL=="ttyUSB*", ATTRS{serial}=="Reader N",  SYMLINK+="SmartReader+_NL"

And now we reenter both our readers into our system.

so for example we take CCcam.cfg

SERIAL READER: /dev/Smartreader+_1702  smartreader+
SMARTCARD CLOCK FREQUENCY: /dev/ Smartreader+_1702 80000000
SERIAL READER: /dev/Smartreader+_NL  smartreader+
SMARTCARD CLOCK FREQUENCY: /dev/ Smartreader+_NL 60000000

that’s it folkz!!