Feed Hunters Guide January 2023 Fully Revised & Updated,

European Beam 53 east to 45 west

The Active part of the Ku band where Feeds can be

found in the UK & Western Europe by William-1

(Polarity indicated when only one is in use)

--------------------------------------------------------------------------------------------------------------------------------------

53°East Express AM6

11450 to 11500

12500 to 12670

52°E TürkmenÄlem/MonacoSat

11260 to 11500

12200 to 12500

47.7°E Eutelsat Quantum

11200 to 11450

11700 to 12750

46°E AzerSpace-1

10970 to 11100

11150 to 11180 V

42°E Türksat 3A/4A

10950 to 11161

11455 to 11521

11591 to 11700

11980 to 12060

12500 to 12750

39°E Hellas Sat-3

12600 to 12650

39°E Hellas Sat-4

10950 to 11200 V

36°E Eutelsat 36B

11050 to 11105 V

11640 to 11660 V

12550 to 12750

33.1°E Eutelsat 33E

10950 to 11180

11600 to 11680 V

12500 to 12600 V

30.9°E Türksat 5A

10950 to 11200

11450 to 11700

28.2°E Astra 2F

12500 to 12750

26°E Badr-4

12530 to 12750

23.5°E Astra 3B

11450 to 11750

12500 to 12750

21.5°E Eutelsat 21B

10950 to 11180

11450 to 11700

12500 to 12750

19.2°E  Astra 1L & 1N

11450 to 11500 V

11560 to 11670 V

11600 to 11650 H

16°E Eutelsat 16A

10950 to 11200

11660 to 11700 V

12500 to 12750

13°E Hotbird 13B

12625 to 12690 V

13°E Hotbird 13C

12695 to 12750 V

10°E Eutelsat 10A

10700 to 10950 V

10950 to 11700

12500 to 12750

7°E Eutelsat 7B

10950 to 11200

11590 to 11700 V

12500 to 12750

7°E Eutelsat 7C

11495 to 11690 V

4.8°E Astra 4A

12500 to 12750

3.1°E Eutelsat 3B

10950 to 11190 H

11450 to 11700

12500 to 12750

1.9°E BulgariaSat-1

11700 to 12500

1°West Intelsat 10-02

10950 to 11190 V

11450 to 11700 V

12510 to 12545 V

3.0°W ABS 3A

10960 to 11120 V

11450 to 11700 V

12500 to 12680 V

5.0°W Eutelsat 5 West B

11040 to 11200

11450 to 11600 H

8.0°W Eutelsat 8 West B

12500 to 12750 V

11.0°W Express AM44

11050 to 11200 H Europe

11450 to 11690 H Europe

15°W Telstar 12

11600 to 11700

12500 to 12650

22°W SES 4

10960 to 11170 H

11550 to 11700 H

12600 to 12660 H

27.5°W Intelsat 901

11140 to 11180 V

30°W Hispasat 30W-5/30W-6

11490 to 11700

11950 to 12050

12570 to 12740

34.5°W Intelsat 35e

10950 to 11180

11545 to 11690

36°W Hispasat 36W-1

11450 to 12200

12500 to 12750

37.6°W Telstar 11N

10950 to 11060

11550 to 11700

12500 to 12750

45.0°W Intelsat 14

11500 to 11680